### OBJECTIVES:

- To create buzzer sound on Project Board.

- To do I/O bit programming in HCS12 C language.

### REFERENCE:

- Mazidi and Causey “HCS12 Microcontroller and Embedded Systems,” Chapter 7.

### MATERIALS:

- CodeWarrior

- Project Board with HCS12 MCU Module

### ACTIVITY 1

Modify the Toggle program in Lab 7A to toggle only a single bit of PORTA. As we discussed in Lab7A, the bits of PORTA are connected to the LEDs. Run the program and verify that the LED of PORTA is toggling. By changing the R3 value in the DELAY subroutine make the time delay smaller or longer and run the program to see the effect.

### ACTIVITY 2

On Project Board, the PT0 bit is connected to a buzzer.  By toggling the PT0 you can sound the buzzer. Download and run the sample program on the www.MicroDigitalEd.com site under Project Board Sample Programs. The program is called “Sounding the Buzzer of the Project Board in C”. Change the value for MsDelay(n) subroutine to make different sounds.

### ACTIVITY 3

Modify the program in Activity 2 to use 8 bits of DIP switches on PORTB to decide the delay size. Now, run the program and change the values on DIP switches to hear different sounds on the buzzer.

Name:						Date: 				Class: