### OBJECTIVES:

- To write a program to convert binary to ASCII on Project Board.

### REFERENCE:

- Mazidi and Causey “HCS12 Microcontroller and Embedded Systems,” Chapter 7.

### MATERIALS:

- CodeWarrior

- Project Board with HCS12 Module

### ACTIVITY 1

Write a C program to get a byte of binary data from PORTB switches after converting it to ASCII display the result on LEDs of PORTA. Place a 2-second delay in between each of three ASCII showing so you can examine the result.  Run the program to verify it works.

### ACTIVITY 2

Write a program to convert the following ASCII to binary and display the result on LEDs of PORTA.

unsigned ascii_1 =0x31;		// Least significant digit

unsigned ascii_2 = 0x39;		; Mid digit

unsigned ascii_3 = 0x35;		; Most significant digit

Change the above number and verify the result.

Name:						Date: 				Class:

- Find the value z after the following code:

unsigned char z=0x55;

z=z^0xff;

z =		in hex

- Find the value z after the following code:

unsigned char z=0xaa;

z=z^0xff;

z =		in hex

- Find the value z after the following code.

unsigned char z=255;

z=z%10;

z =		in hex

- Find the value z after the following code:

unsigned char z=0x99;

z=z>>4;

z =		in hex

- Find the value z after the following code.

unsigned char z=0x82;

z=z<<4;

z =		in hex