### OBJECTIVES:

- To experiment with the concept of look-up table in C program.

### REFERENCE:

- Mazidi and Causey “HCS12 Microcontroller and Embedded Systems,” Chapter 7.

### MATERIALS:

- CodeWarrior

- Project Board with HCS12 MCU Module

### ACTIVITY 1

Using look-up table, write a C program to the get value of X from DIP switches of PORTB and send the Y to LEDs of PORTA. Y=X^2 + 4X +5. Use the lower 4 bits of DIP switches for X. Run the program to verify it works.

### ACTIVITY 2

Using look-up table, write a program to the get 4-bit binary (between 0000 – 1111) from DIP switches of PORTB and send ASCII value for it to LEDs of PORTA. Use the lower 4 bits of DIP switches for binary input. Run the program to verify it works.

Name:						Date: 				Class: