### OBJECTIVES:

- To interface a DAC to the PIC.

- To generate a sine wave on the scope using the DAC.

### REFERENCES:

- Mazidi, McKinlay, and Causey,  “PIC Microcontroller and Embedded Systems,” Chapter 13.

- www.national.com/catalog/AnalogDataAcquisition.html

### MATERIALS:

- PIC Trainer

- MPLAB

- DAC module

### ACTIVITY

First generate (calculate) a table similar to Table 13-5 of the textbook for sine values of 4-degree increments. Then connect the DAC module to the PIC Trainer or set up a circuit like the one shown in Figure 13-11. Write and run a program to create the sine wave on the oscilloscope. Put a delay in between each outputting of values to DAC and observe the sine wave on the scope. Change the delay values and state your conclusions.

Name:						Date: 				Class:

1.  Define the following terminology in DAC.

(a) resolution

(b) full-scale voltage output

(c) settling time

2. For your circuit, find Vout for the following inputs.

(a) 11001100

(b) 10001111

3.  To get a smaller step size, we need DAC with ________ (more, less) data bit inputs.

4.  In Figure 13-11 of the textbook, assume that R = 5 KOhms. Calculate Vout for the

following binary inputs.

(a) 11000010

(b) 01000001

(c) 00101100

(d) 11111111