### OBJECTIVES:

- To program the timer interrupt of the PIC.

- To program the serial interrupt of the PIC.

- To program the hardware interrupt of the PIC.

### REFERENCE:

- Mazidi, McKinlay, and Causey, “PIC Microcontroller and Embedded Systems,” Chapter 11.

### MATERIALS:

- PIC Trainer

- MPLAB

- Square wave generator

### ACTIVITY 1

Write a program to get data from a single bit of RB1 and send it to RB7 continuously while an interrupt will do the following: One of the timers will toggle pin RB2 every 100 microseconds.  In place of RB1,RB2, and RB7 pins you can use ports of your own choice.

### ACTIVITY 2

Add the following interrupt to Activity 1: A serial interrupt service routine will receive data from a x86 PC HyperTerminal  and displays it on LEDs connected to one of the ports.

### ACTIVITY 3

Add the following interrupt to Activity 2: Connect one of the hardware interrupt pins to a normally high debounce SW and any time the switch is pressed, a single LED is turned on.

### ACTIVITY 4

Add the following interrupt to Activity 3: A square wave is connected to hardware interrupt and any time a L-to-H pulse comes in, a single LED is turned on. The rate of "On" and "Off" is the same as the square wave frequency.

Name:						Date: 				Class:

1.  For the PIC18F458/4580, name all the interrupts associated with hardware, timers and Serial port.

2. In Timer0 16-bit mode, indicate when TMR0IF causes the interrupt.

3. In Timer0 8-bit mode, indicate when TMR0IF causes the interrupt.

4. On reset, all hardware interrupts are _________ (edge, level) triggered.

5. True or False. On reset, high priority is assigned to all interrupts.

6. True or False. There is only a single interrupt for the serial data transfer.