### OBJECTIVES:

- To examine and use MPLAB assembler.

- To examine and use MPLAB simulator.

### REFERENCE:

- Mazidi, McKinlay, Causey “PIC Microcontroller and Embedded Systems,” Chapter 2.

### MATERIALS:

- Download MPLAB assembler/simulator and its Tutorial from the following websites:

### WEB SITES:

www.Microchip.com		for MPLAB

www.MicroDigitalEd.com	for MPLAB Tutorial

### ACTIVITY 1

Write and assemble a program to move value 99H to WREG register. Then from register WREG move it to FileReg locations  20H – 27H. Use the MPLAB simulator to single-step the program and examine the fileRegs.

### ACTIVITY 2

Write and assemble a program to add all the single digits of your ID number and save the result in WREG. Pick 7 random numbers (all single digit) if you do not want to use your ID number. Then use the MPLAB simulator to single-step the program and examine the registers.

Name:						Date: 				Class:

1.  Indicate the size (8- or 16-bit) of each of the following registers.

PC =			WREG=			FileReg Location 0=

FileReg location 1=			FileReg location 20H=		PORTB=

2. For Question 1, indicate the largest value (in decimal) that each register can contain.

PC =				WREG=			FileReg Location 0=

FileReg location 1=		FileReg location 20H=	PORTB=

3. For Question 1, indicate the largest value (in hex) that each register can contain.

PC =				WREG=			FileReg Location 0=

FileReg location 1=		FileReg location 20H=	PORTB=

4. Who generates each of the following files and what is the use of each.

.asm

.lst

.o

.err

.hex