### OBJECTIVES:

- To examine the flag bits of status register.

### REFERENCE:

- Mazidi, McKinlay, and Causey “PIC Microcontroller and Embedded Systems,” Chapter 2.

### MATERIALS:

- MPLAB assembler and simulator.

### ACTIVITY 1

Write and assemble a program to add the following data and then use the simulator to examine the C, DC and Z flags after the execution of each addition.

99H, 01, 66H, 89H, 97H, 35H

### ACTIVITY 2

Write and assemble a program to load a value into each location of fileRegs 0x20 – 0x24. Use the COMF instruction to complement the value in each location. Use the simulator to single-step and examine the flags and fileReg content after the execution of each instruction.

Name:						Date: 				Class:

1. Find the value of the C, DC and Z flags after the execution of  the following code.

MOVLW 0x85

ADDLW 0x9F

2. Find the value of the C, DC and Z flags after the execution of  the following code.

MOV LW 0x99

ADDLW 0x67

ADDLW 0xFF

3. Find the value of the C, DC and Z flags after the execution of  the following code.

MOV LW 0xAA

ADDLW  0x55

4. Find the value of the C, DC and Z flags after the execution of  the following code.

MOVLW 0xFE

ADDLW 0x1

ADDLW 0x1

5. Upon reset, what is the value in the PC register?

6. Give the address of WREG register.